Appendix A

SCHEDULES OF REQUIREMENTS, PARTICULARS AND GUARANTEES

a1.	FOREWORD

These Schedules of Requirements, General Particulars and Guarantees form a part of the Specification.

These Schedules do not purport to include all the necessary provisions of a Contract.

Except where otherwise specified or implied herein, transformers shall comply with the requirements, as applicable, of the latest amendments of the Standards of the International Electrotechnical Commission (IEC) or British Standards (BS) where specified and in particular the standards listed in Section 19 of this Specification.

No departures from the Specification except those shown in the Schedule J of Departures shall be made without the written approval of Electricity North West.

SCHEDULE A

GENERAL PARTICULARS OF DEFINITE WORK

(Relevant details to be completed by Electricity North West)

| 1 | Section(s) for purposes of payments and taking over | See commercial contract |
| --- | --- | --- |
| 2 | Substation Name |  |
| 3 | Number of 132kV/Lower Voltage Transformers required |  |
| 4 | Continuous maximum rating |  |
|  | (a) Rating MVA |  |
|  | (b) Power Factor |  |
|  | (c) Voltage kV |  |
| 5 | Minimum continuous ONAN rating MVA |  |
| 6 | Rated higher voltage between phases kV |  |
| 7 | Rated lower voltage between phases kV (not applicable to reactors) |  |
| 8 | Whether a standby voltage control panel is required | Yes/No |
| 9 | Number of Earthing/Auxiliary Transformers required |  |
| 10 | Type and Rating of Earthing Transformer and Neutral Earthing System - | Low Reactance - 30s, 1000A, Resistance Earthed |
| 11 | Continuous maximum rating kVA |  |
| 12 | Rated higher voltage between phases kV |  |
| 13 | Rated lower voltage between phases (no load kV) | 0.400/0.230, three-phase, four wire |
| 14 | Purchaser's Nomenclature |  |
|  | (a) Main Unit | GT1 |
|  | (b) Earthing/Auxiliary Transformer | EAT1 |
| 15 | CT Details | As indicated in diagram overleaf |

SCHEDULE A

GENERAL PARTICULARS OF DEFINITE WORK

(Relevant details to be completed by Electricity north West)

DIAGRAM OF CT DETAILS

| HV |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | C |  |  |  |  |  |  |  | B |  |  |  |  |  |  |  | A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| P2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Function |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ratio |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N |  |  |  |  |  |  |  |  |  |  |  |  |
| Characteristic |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | P1 |  |  |  |  |  |  |  |
| Function |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Function |  |  |  |  |  |  |  |  |  |  |  |  |
| Ratio |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ratio |  |  |  |  |  |  |  |  |  |  |  |  |
| Characteristic |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Characteristic |  |  |  |  |  |  |  |  |  |  |  |  |
| Function |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Function |  |  |  |  |  |  |  |  |  |  |  |  |
| Ratio |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ratio |  |  |  |  |  |  |  |  |  |  |  |  |
| Characteristic |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Characteristic |  |  |  |  |  |  |  |  |  |  |  |  |
| P1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | P2 |  |  |  |  |  |  |  |
| P1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ratio |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Function |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Characteristic |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ratio |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Function |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Characteristic |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| P2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | C |  |  |  |  |  |  |  | b |  |  |  |  |  |  |  | a |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LV |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note. Diagram does not show any CTs for internal use, for example WTIs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

SCHEDULE B

COMMENCEMENT DATE AND DATES OF READINESS FOR INSPECTION,

TESTING, DELIVERY, ACCESS TO SITE AND COMPLETION

DEFINITE WORK

(To be completed by Tenderer)

For UU’s Programme see Commercial Contract

Commencement Date:………………………….

Section(s) for purposes of payments and taking over Site Station. Times are calculated from Commencement Date and are given in calendar months.

|  | Time from Commencement | Number of Calendar Months |
| --- | --- | --- |
| 1 | Within which detailed drawings shall be submitted for approval |  |
| 2 | Within which the material shall be ready for inspection and testing |  |
| 3 | Within which the Contractor will require access to the site |  |
| 4 | Within which the material shall be delivered to site |  |
| 5 | Within which the works shall be completed, tested and ready for continuous use |  |

Completion date:……………………………..

SCHEDULE C.1

MANUFACTURERS AND PLACES OF MANUFACTURE, TESTING AND INSPECTION

132KV/LOWER VOLTAGE TRANSFORMER

(Relevant details to be completed by Tenderer)

| Item | Manufacturer's Drawing Number and/or Type Designation | Manufacturer | Place of Manufacture | Place of Testing and/or Inspection |
| --- | --- | --- | --- | --- |
| Transformer Complete |  |  |  |  |
| On-load Tap-Change equipment |  |  |  |  |
| On load Tap-changer porcelain |  |  |  |  |
| HV Bushings |  |  |  |  |
| LV Bushings |  |  |  |  |
| Neutral Bushings |  |  |  |  |
| Insulating Cylinders |  |  |  |  |
| Core Plate |  |  |  |  |
| Copper/Aluminium |  |  |  |  |
| Steel Tanks |  |  |  |  |
| Radiators |  |  |  |  |
| Oil Pumps |  |  |  |  |
| Oil Pump Motors |  |  |  |  |
| Oil Flow Indicators |  |  |  |  |
| Fans |  |  |  |  |

Continued overleaf

SCHEDULE C.1

MANUFACTURERS AND PLACES OF MANUFACTURE, TESTING AND INSPECTION

132KV/LOWER VOLTAGE TRANSFORMER

(Relevant details to be completed by Tenderer)

| Item | Manufacturer's Drawing Number and/or Type Designation | Manufacturer | Place of Manufacture | Place of Testing and/or Inspection |
| --- | --- | --- | --- | --- |
| Fan Motors |  |  |  |  |
| Gaskets for Oil Tight Joints |  |  |  |  |
| Pressure Relief Device |  |  |  |  |
| Regenerative Breather |  |  |  |  |
| Gas & Oil Actuated Relay(s) |  |  |  |  |
| Marshalling Kiosk |  |  |  |  |
| Temperature Indicating Devices |  |  |  |  |
| Motor Control Equipment |  |  |  |  |
| Alarm Devices |  |  |  |  |
| Standby Control Panel |  |  |  |  |
| AVR Relays |  |  |  |  |
| Material for Anti-vibration Mountings |  |  |  |  |

Any deviation from this Schedule shall be notified in writing as soon as possible for Electricity North West’ approval.

Any deviation or failure during manufacturing and tests shall be notified by the Contractor to Electricity North West.

SCHEDULE C.2

MANUFACTURERS AND PLACES OF MANUFACTURE, TESTING AND INSPECTION

EARTHING/AUXILIARY TRANSFORMER

(Relevant details to be completed by Tenderer)

| Item | Manufacturer's Drawing Number and/or Type Designation | Manufacturer | Place of Manufacture | Place of Testing and/or Inspection |
| --- | --- | --- | --- | --- |
| Earth/Aux. Transformer Complete |  |  |  |  |
| HV Bushings |  |  |  |  |
| LV Bushings |  |  |  |  |
| Neutral Bushings |  |  |  |  |
| Insulating Cylinders |  |  |  |  |
| Core Plate |  |  |  |  |
| Copper/Aluminium |  |  |  |  |
| Steel Tanks |  |  |  |  |
| Radiators |  |  |  |  |
| Oil |  |  |  |  |
| Oil Valves |  |  |  |  |
| Gaskets for oil Tight Joints |  |  |  |  |
| Pressure Relief Device |  |  |  |  |
| Regenerative Breather |  |  |  |  |

SCHEDULE C.2

MANUFACTURERS AND PLACES OF MANUFACTURE, TESTING AND INSPECTION

EARTHING/AUXILIARY TRANSFORMER

(Relevant details to be completed by Tenderer)

| Item | Manufacturer's Drawing Number and/or Type Designation | Manufacturer | Place of Manufacture | Place of Testing and/or Inspection |
| --- | --- | --- | --- | --- |
| Gas and Oil Actuated Relay(s) |  |  |  |  |
| Marshalling Kiosk |  |  |  |  |
| Temperature Indicating Devices |  |  |  |  |
| Transformer Fuse-switch |  |  |  |  |

Any deviation from this Schedule shall be notified in writing as soon as possible for Electricity North West’ approval.

Any deviation or failure during manufacturing and tests shall be notified by the Contractor to Electricity North West.

SCHEDULE D

REQUIREMENTS FOR 132KV/LOWER VOLTAGE TRANSFORMERS

PART 1

CONTROL, OPERATION, INDICATIONS, ALARMS AND TRIPS

(To be completed by Electricty North West)

For ……………………………………

| The requirements for control, operation, indications, alarms and trips are detailed in the following Electricity North West drawings: |  |
| --- | --- |
| UU Drawing No. | Provided with Tender |
| 900200-19 | Yes/No |
| 900200-20 | Yes/No |
| 900200-21 | Yes/No |

Loose Control, Operation, Indication, Alarm and Trip apparatus for mounting and wiring under a separate contract shall be supplied to:   (to be completed by Electricity North West).

SCHEDULE D

REQUIREMENTS FOR 132KV/LOWER VOLTAGE TRANSFORMERS

PART II

(To be completed by Electricty North West)

For ……………………………………..

| Item No. | Description |  |
| --- | --- | --- |
|  | Rating and Performance |  |
| 1 | Continuous Maximum Rating (CMR) at rated voltage MVA |  |
| 2 | Continuous ONAN rating with mixed cooling MVA |  |
| 3 | No-load voltage ratio at nominal tap kV |  |
| 4 | Winding Connections: |  |
|  | (a) HV windings |  |
|  | (b) LV windings |  |
|  | (c) IEC Vector Group symbols |  |
|  | (d) Whether links are required for alternative vector group | Yes/No |
| 5 | Impedance voltage at 75°C and CMR (to be completed for impedance on normal tapping or envelope impedances as appropriate) | See Fig D1 |
| 6 | Short Circuit capacity: |  |
|  | (a) Available at HV terminals MVA |  |
|  | (b) Available at LV terminals MVA |  |
|  | Voltage Control |  |
| 7 | Method of variation of voltage radio | On load |
| 8 | Total range of variation of voltage ratio (Expressed as a percentage of ....... with .......... constant): |  |
|  | Plus per cent |  |
|  | Minus per cent |  |
| 9 | Size of step % |  |
| 10 | Whether control by direct wire or selective system required at remote control point | Yes/No |

SCHEDULE D

REQUIREMENTS FOR 132KV/LOWER VOLTAGE TRANSFORMERS

PART II

(To be completed by Electricity North West)

For ………………………………..

| ItemNo | Description |  |
| --- | --- | --- |
| 11 | Automatic voltage control: |  |
|  | (a) Whether required | Yes/No |
|  | (b) Whether group parallel control is required | Yes/No |
| 12 | Space available for line drop compensation current transformer in LV switchgear | Yes/No |
|  | (a) Inside diameter mm |  |
|  | (b) Outside diameter mm |  |
|  | (c) Axial length including clamps mm |  |
| 13 | Whether load shedding facilities are required, either by direct wire or selective system at remote control point | Yes/No |
| 14 | Number of Marshalling kiosk (cubicle) compartments |  |
| 15 | Marshalling kiosk (cubicle) switch-sockets: | 240V a.c. |
|  | (a) Type |  |
|  | (b) Voltage |  |
| 16 | Marshalling kiosk (cubicle) telephone jacks required: | Yes/No |
|  | (a) Type |  |
|  | Terminations |  |
| 17 | Terminals: |  |
|  | (a) Bushing insulators or cable boxes for line terminals: |  |
|  | i) HV |  |
|  | ii) LV |  |
|  | (b) Bushing insulators or cable boxes for neutral terminals: |  |
|  | i) HV |  |
|  | ii) LV |  |
| 18 | Co-ordinating arc gaps, on tank or outrigger |  |

SCHEDULE D

REQUIREMENTS FOR 132KV/LOWER VOLTAGE TRANSFORMERS

PART II

(To be completed by Electricity North West)

For ………………………………….

| Item No | Description |  |
| --- | --- | --- |
| 19 | Cable Box Details: |  |
|  | (a) Higher voltage: |  |
|  | i) Number of poles |  |
|  | ii) Number of cables per phase |  |
|  | iii) Type of cable |  |
|  | iv) Sectional area of conductor mm² |  |
|  | v) Diameter over metal sheathing mm |  |
|  | vi) Diameter over armouring |  |
|  | vii) Special particulars, if any (e.g. whether gland or island layer insulation is required). |  |
|  | viii) Whether cables enter box from below or from above |  |
|  | ix) Filling medium for cable box |  |
|  | (b) Higher voltage neutral point: |  |
|  | i) Number of poles |  |
|  | ii) Number of cables per phase |  |
|  | iii) Type of cable |  |
|  | iv) Sectional area of conductor mm² |  |
|  | v) Diameter over metal sheathing mm |  |
|  | vi) Diameter over armouring mm |  |
|  | vii) Special particulars, if any(e.g. whether gland or island layer insulation is required). |  |
|  | viii)Whether cables enter box from below or from above |  |
|  | ix) Filling medium for cable box |  |

SCHEDULE D

REQUIREMENTS FOR 132KV/LOWER VOLTAGE TRANSFORMERS

PART II

(To be completed by Electricity North West)

For ………………………………..

| ItemNo | Description |  |
| --- | --- | --- |
|  | (c) Lower voltage: |  |
|  | i) Number of poles |  |
|  | ii) Number of cables per phase |  |
|  | iii) Type of cable |  |
|  | iv) Sectional area of conductor mm² |  |
|  | v) Diameter over metal sheathing mm |  |
|  | vi) Diameter over armouring mm |  |
|  | vii) Special particulars, if any (e.g. whether gland or island layer insulation is required) |  |
|  | viii) Whether cables enter box from below or from above |  |
|  | ix) Filling medium for cable box |  |
|  | (d) Lower voltage neutral point |  |
|  | i) Number of poles |  |
|  | ii) Number of cables per phase |  |
|  | iii) Type of cable |  |
|  | iv) Sectional area of conductor mm² |  |
|  | v) Diameter over metal sheathing mm |  |
|  | vi) Diameter over armouring mm |  |
|  | vii) Special particulars, if any (e.g. whether gland or island layer insulation is required). |  |
|  | viii)Whether cables enter box from below or from above |  |
|  | ix) Filling medium for cable box |  |
| 20 | Whether CT accommodation is required on bushing insulators: |  |
|  | (a) HV | Yes/No |
|  | (b) LV | Yes/No |

SCHEDULE D

REQUIREMENTS FOR 132KV/LOWER VOLTAGE TRANSFORMERS

PART II

(To be completed by Electricty North West)

For ………………………………….

| Item No | Description |  |
| --- | --- | --- |
|  | Cooling |  |
| 21 | Cooling designation |  |
| 22 | Arrangement of coolers: |  |
|  | (a) Tank attached | Yes/No |
|  | (b) Separate free-standing | Yes/No |
| 23 | Whether valves for oil filtering require flexible hose adaptors | Yes/No |
| 24 | Size of flexible hose adaptor | 50mm |
|  | General |  |
| 25 | Whether anti-vibration pads are required | Yes/No |
| 26 | Whether a noise enclosure is required from the outset | Yes/No |
| 27 | Number of copies of Operating and Maintenance Instructions required per station (site) | One draft, two final per station |
| 28 | Transformer Management System Required | Yes/No |

FIGURE D1 - ENVELOPE OF IMPEDANCE CHARACTERISTIC

24.1

21.7

27.8

20.6

Impedance voltage %

at 75oC to 90 MVA base

Band of possible values

for 90 MVA transformer.

16.0

14.5

18.5

13.8

Impedance voltage %

at 75oC to 60 MVA base

Band of possible values

for 60 MVA transformer.

Tapping %

- 20%

+ 10%

9.4

8.4

10.8

7.5

Impedance voltage %

at 75oC to 30 MVA base

Band of possible values

for 30 MVA transformer.

SCHEDULE D

REQUIREMENTS FOR EARTHING/AUXILIARY TRANSFORMERS

PART III

(To be completed by Electricty North West)

For ……………….………………….

| Item No | Description |  |
| --- | --- | --- |
|  | Rating and Performance |  |
| 1 | Method of Neutral Earthing transformer combined with liquid earthing resistor supplied by Electricity North West | Yes/No |
| 2 | Continuous Maximum Rating (CMR) at rated voltage kVA | 200 |
| 3 | No-load voltage ratio at normal tap kV |  |
| 4 | Winding Connections: |  |
|  | (a) HV windings |  |
|  | (b) LV windings |  |
|  | (c) IEC 76-4 (BS 171/4) Vector Group symbols |  |
|  | (d) Whether links are required for alternative vector group | Yes/No |
| 5 | Impedance voltage at 75°C and CMR (to be completed for impedance on normal tapping) | See Fig D1 |
| 6 | Terminals: |  |
|  | (a) Bushing insulators or cable boxes for line terminals: |  |
|  | i) HV |  |
|  | ii) LV |  |
|  | (b) Bushing insulators or cable boxes for neutral terminals: |  |
|  | i) HV |  |
|  | ii) LV |  |

SCHEDULE D

REQUIREMENTS FOR EARTHING/AUXILIARY TRANSFORMERS

PART III

(To be completed by Electricity North West)

For …………………………………….

| Item No | Description |  |
| --- | --- | --- |
| 7 | Cable Box Details: |  |
|  | (a) Higher voltage: |  |
|  | i) Number of poles |  |
|  | ii) Number of cables per phase |  |
|  | iii) Type of cable |  |
|  | iv) Sectional area of conductor mm² |  |
|  | v) Diameter over metal sheathing mm |  |
|  | vi) Diameter over armouring mm |  |
|  | vii) Special particulars, if any (e.g. whether gland or island layer insulation is required). |  |
|  | viii) Whether cables enter box from below or from above |  |
|  | ix) Filling medium for cable box |  |
|  | (b) Higher voltage neutral point: |  |
|  | i) Number of poles |  |
|  | ii) Number of cables per phase |  |
|  | iii) Type of cable |  |
|  | iv) Sectional area of conductor mm² |  |
|  | v) Diameter over metal sheathing mm |  |
|  | vi) Diameter over armouring mm |  |
|  | vii) Special particulars, if any (e.g. whether gland or island layer insulation is required). |  |
|  | viii) Whether cables enter box from below or from above |  |
|  | ix) Filling medium for cable box |  |

SCHEDULE D

REQUIREMENTS FOR EARTHING/AUXILIARY TRANSFORMERS

PART III

(To be completed by Electricity North West)

For ……………………………………

| Item No | Description |  |
| --- | --- | --- |
| 7 | (c) Lower voltage: |  |
|  | i) Number of poles |  |
|  | ii) Number of cables per phase |  |
|  | iii) Type of cable |  |
|  | iv) Sectional area of conductor mm² |  |
|  | v) Diameter over metal sheathing mm |  |
|  | vi) Diameter over armouring mm |  |
|  | vii) Special particulars, if any (e.g. whether gland or island layer insulation is required). |  |
|  | viii) Whether cables enter box from below or from above |  |
|  | ix) Filling medium for cable box |  |
|  | (d) Lower voltage neutral point: |  |
|  | i) Number of poles |  |
|  | ii) Number of cables per phase |  |
|  | iii) Type of cable |  |
|  | iv) Sectional area of conductor mm² |  |
|  | v) Diameter over metal sheathing mm |  |
|  | vi) Diameter over armouring mm |  |
|  | vii) Special particulars, if any (e.g. whether gland or island layer insulation is required). |  |
|  | viii) Whether cables enter box from below or from above |  |
|  | ix) Filling medium for cable box |  |
| 8 | Cooling designation | Yes/No |
| 9 | Arrangement of coolers: Tank attached | Yes/No |
| 10 | Whether valves for oil filtering require flexible hose adpators | Yes/No |
| 11 | Size of flexible hose adaptors | 25mm |

SCHEDULE D

REQUIREMENTS FOR EARTHING/AUXILIARY TRANSFORMERS

PART III

(To be completed by Electricity North West)

For ……………………………………

| Item No | Description |  |
| --- | --- | --- |
|  | General |  |
| 12 | Whether anti-vibration pads are required | Yes/No |
| 13 | Whether a noise enclosure is required from the outset | Yes/No |
| 14 | Number of copies of Operating and Maintenance Instructions required per station (site) | One draft, two final per station |

SCHEDULE E

TECHNICAL PARTICULARS AND GUARANTEES

PART I

(To be completed by Tenderer)

FOR 132KV/LOWER VOLTAGE TRANSFORMERS

| Item No | Description |  |
| --- | --- | --- |
|  | Magnetic Circuit |  |
| 1 | Core Construction: |  |
|  | (a) Taped/banded/bolted limbs |  |
|  | (b) Taped/banded/bolted yokes |  |
|  | (c) Taping/banding material (as applicable) |  |
|  | (d) Number of limbs |  |
|  | (e) Number of limbs wound |  |
| 2 | Insulation of: |  |
|  | (a) Core bolts |  |
|  | (b) Core bolt washers |  |
|  | (c) Side plates |  |
|  | (d) Core laminations |  |
| 3 | Flux density in iron: |  |
|  | Nominal value at normal voltage, frequency and normal tap: |  |
|  | i) Wound limbs tesla |  |
|  | ii) Yokes tesla |  |
|  | iii) Unwound limbs tesla |  |
|  | iv) Shields tesla |  |
|  | Maximum value at nominal voltage, minimum frequency and minimum tap |  |
|  | i) Wound limbs tesla |  |
|  | ii) Yokes tesla |  |
|  | iii) Unwound limbs tesla |  |
|  | iv) Shields tesla |  |
| 4 | Whether tank or other flux shields are incorporated | Yes/No |

SCHEDULE E

TECHNICAL PARTICULARS AND GUARANTEES

PART I

(To be completed by Tenderer)

FOR 132KV/LOWER VOLTAGE TRANSFORMERS

| Item No | Description |  |
| --- | --- | --- |
|  | Windings |  |
| 5 | Types and arrangement of: |  |
|  | (a) HV windings |  |
|  | (b) LV windings |  |
|  | (c) Tapping windings (as applicable) |  |
|  | (d) Winding arrangement, i.e. core/...../...../ |  |
| 6 | Arrangement of tappings (linear, coarse/fine, reversing) |  |
| 7 | Conductor material for: |  |
|  | (a) HV windings |  |
|  | (b) LV windings |  |
|  | (c) Tapping windings |  |
| 8 | Conductor insulation: |  |
|  | (a) HV windings |  |
|  | (b) LV windings |  |
|  | (c) Tapping windings |  |
| 9 | Oil circulation (i.e. natural/partially directed/directed): |  |
|  | (a) to windings |  |
|  | i) HV windings |  |
|  | ii) LV windings |  |
|  | iii) Tapping windings |  |
|  | (b) through windings |  |
|  | i) HV windings |  |
|  | ii) LV windings |  |
|  | iii) Tapping windings |  |

SCHEDULE E

TECHNICAL PARTICULARS AND GUARANTEES

PART I

(To be completed by Tenderer)

FOR 132KV/LOWER VOLTAGE TRANSFORMERS

| Item No | Description |  |
| --- | --- | --- |
| 10 | Short circuit capability: |  |
|  | (a) Potential axial thrust for worst fault condition of each winding: |  |
|  | i) HV windings Tonne |  |
|  | ii) LV windings Tonne |  |
|  | iii) Tapping windings Tonne |  |
| 11 | (b) Coil clamping short circuit withstand capacity |  |
|  | i) HV windings Tonne |  |
|  | ii) LV windings Tonne |  |
|  | iii) Tapping windings Tonne |  |
| 12 | Current density in windings (at normal tapping position): |  |
|  | (a) HV windings (CMR) A/mm² |  |
|  | (b) LV windings (CMR) A/mm² |  |
|  | (c) Tapping windings (as applicable) (CMR) A/mm² |  |
|  | Performance Characteristics |  |
| 13 | No-load loss at normal ratio (excluding cooling plant loss) kW |  |
| 14 | Cooling plant loss kW |  |
| 15 | Load losses at CMR and 75°C |  |
|  | (a) On normal tapping: HV/LV kW |  |
|  | (b) On tapping for maximum loss - Tap position number HV/LV kW |  |
| 16 | Impedance voltage at 75°C and CMR |  |
|  | (a) Impedance on normal tapping: HV/LV % |  |
|  | (b) Minimum impedance on minimum tapping - Tap position number HV/LV % |  |
|  | (c) Maximum impedance on minimum tapping: HV/LV % |  |
|  | (d) Minimum impedance on maximum tapping - Tap position number |  |
|  | (e) Maximum impedance on maximum tapping: HV/LV % |  |

SCHEDULE E

TECHNICAL PARTICULARS AND GUARANTEES

PART I

(To be completed by Tenderer)

FOR 132KV/LOWER VOLTAGE TRANSFORMERS

| Item No | Description |  |
| --- | --- | --- |
| 17 | Demonstrable zero phase sequence impedance at 75°C and on normal tapping, referred to HV side, at 10% full load current in each phase winding: HV/LV ohms per phase |  |
| 18 | Calculated zero phase sequence impedance at 75°C and on normal tapping, referred to HV side, assuming rated voltage (single phase) applied between line terminals and neutral: |  |
|  | (a) Z Ω per phase (approx.) |  |
|  | (b) Z Ω per phase (approx.) |  |
|  | (c) Z Ω per phase (approx.) |  |
|  | (d) Z Ω per phase (approx.) |  |
| 19 | Hottest spot winding temperature on most onerous tap position. (Average ambient air temperature 30°C or water temperature at 20°C) Position No. |  |
|  | (a) At CMR |  |
|  | i) HV °C |  |
|  | ii) LV °C |  |
| 20 | Maximum observable oil temperature (Average ambient air temperature 30°C) |  |
|  | (a) At CMR |  |
|  | i) Top oil °C |  |
|  | ii) At inlet to cooler °C |  |
|  | iii) At outlet to cooler °C |  |
|  | (b) At ONAN - top oil |  |
|  | (c) With HV at CMR. LV loaded 0.9 pf lagging and tertiary loaded 60 MVAr lagging - top oil °C |  |
| 21 | Proportion of sum of iron and copper losses at CMR which will be supplied during temperature rise test % |  |

SCHEDULE E

TECHNICAL PARTICULARS AND GUARANTEES

PART I

(To be completed by Tenderer)

FOR 132KV/LOWER VOLTAGE TRANSFORMERS

| Item No | Description |  |
| --- | --- | --- |
| 22 | Calculated winding capacitance: |  |
|  | (a) Approximate series capacitance of each phase winding: |  |
|  | i) HV pF |  |
|  | ii) LV pF |  |
|  | (b) Approximate shunt capacitance of each phase winding with core and tank earthed: |  |
|  | i) HV to earth with LV windings unearthed pF |  |
|  | ii) LV to earth with HV windings unearthed pF |  |
|  | (c) Approximate capacitance HV to LV phase winding with LV winding unearthed pF |  |
| 23 | (a) Guaranteed sound power level (ONAN) dBA |  |
|  | (b) Guaranteed sound power level (with cooling in operation) dBA |  |
|  | Tank and Cooler |  |
| 24 | Tank material |  |
| 25 | Thickness of tank: |  |
|  | (a) Sides mm |  |
|  | (b) Base mm |  |
|  | (c) Cover mm |  |
| 26 | Thickness of radiator plates and/or cooling tubes mm |  |
| 27 | Total oil required (including cooling system) litre |  |
| 28 | Volume of oil to be removed: |  |
|  | (a) to gain access to core earthing link litre |  |
|  | (b) to effect in situ change of HV bushing litre |  |
|  | (c) to effect in situ change of LV bushing litre |  |
|  | (d) to effect in situ maintenance of one oil cooler litre |  |

SCHEDULE E

TECHNICAL PARTICULARS AND GUARANTEES

PART I

(To be completed by Tenderer)

FOR 132KV/LOWER VOLTAGE TRANSFORMERS

| Item No | Description |  |
| --- | --- | --- |
| 29 | Total volume of conservator(s) litre |  |
| 30 | Volume of oil in (each) conservator between highest and lowest visible levels litre |  |
| 31 | Height of oil at maximum level in the conservator(s) above pad level m |  |
| 32 | Total number of oil pumps |  |
| 33 | Output of each oil pump under service conditions: |  |
|  | (a) quantity litre/s |  |
|  | (b) head metres head of oil (approx.) |  |
| 34 | Continuous rating of each oil pump motor Shaft kW |  |
| 35 | Starting current of each oil pump motor A |  |
| 36 | Total number of fans |  |
| 37 | Rated normal output of each fan: |  |
|  | (a) quantity metres³/sec |  |
|  | (b) head mm, water gauge (approx.) |  |
| 38 | Nominal diameter of fans mm |  |
| 39 | Speed of fans rpm |  |
| 40 | Continuous rating of each fan motor Shaft kW |  |
| 41 | Starting current of each fan motor A |  |
| 42 | Filling medium for transport |  |
| 43 | Total weight as installed in service, including cooling plant, all fittings and oil |  |
|  | (a) quotation estimate Tonne |  |
|  | (b) final design calculation Tonne |  |
| 44 | Weight arranged for transport: |  |
|  | (a) excluding vehicle Tonne |  |
|  | (b) including vehicle (to be completed for transformers weighing in excess of 200 Tonne) Tonne |  |

SCHEDULE E

TECHNICAL PARTICULARS AND GUARANTEES

PART I

(To be completed by Tenderer)

FOR 132KV/LOWER VOLTAGE TRANSFORMERS

| Item No | Description |  |
| --- | --- | --- |
| 45 | Weight of each individual coolers, complete with oil Tonne |  |
| 46 | Weight of each bushing insulator: |  |
|  | (a) HV kg |  |
|  | (b) LV kg |  |
|  | (c) Neutral kg |  |
| 47 | Weight of silica-gel breather charge (if applicable) kg |  |

SCHEDULE E

TECHNICAL PARTICULARS AND GUARANTEES

PART II

(To be completed by Tenderer)

FOR EARTHING/AUXILIARY TRANSFORMERS

| Item No | Description |  |
| --- | --- | --- |
|  | Magnetic Circuit |  |
| 1 | Core Construction: |  |
|  | (a) Taped/banded/bolted limbs |  |
|  | (b) Taped/banded/bolted yokes |  |
|  | (c) Taping/banding material (as applicable) |  |
|  | (d) Number of limbs |  |
|  | (e) Number of limbs wound |  |
| 2 | Insulation of: |  |
|  | (a) Core bolts |  |
|  | (b) Core bolt washers |  |
|  | (c) Side plates |  |
|  | (d) Core laminations |  |
| 3 | Flux density in iron - Nominal value at normal voltage, frequency and normal tap: |  |
|  | i) Wound limbs tesla |  |
|  | ii) Yokes tesla |  |
|  | iii) Unwound limbs tesla |  |
|  | iv) Shields tesla |  |
|  | Windings |  |
| 4 | Types and arrangements of: |  |
|  | (a) HV windings |  |
|  | (b) LV windings |  |
|  | (c) Winding arrangement, i.e. core/ / / |  |
| 5 | Conductor material for: |  |
|  | (a) HV windings |  |
|  | (b) LV windings |  |

SCHEDULE E

TECHNICAL PARTICULARS AND GUARANTEES

PART II

(To be completed by Tenderer)

FOR EARTHING/AUXILIARY TRANSFORMERS

| Item No | Description |  |
| --- | --- | --- |
| 6 | Conductor insulation: |  |
|  | (a) HV windings (b) LV windings |  |
| 7 | Current density in the interconnected star winding with: |  |
|  | (a) The specified earth fault current (30 second rating) A/mm² |  |
|  | (b) The resultant earth fault current with no external impedance in circuit (3 second rating) A/mm² |  |
| 8 | Current density in windings (at normal tapping position): |  |
|  | (a) HV windings (CMR) A/mm² |  |
|  | (b) LV windings (CMR) A/mm² |  |
|  | Performance Characteristics |  |
| 9 | No-load loss at normal ratio kW |  |
| 10 | Load losses at CMR and 75°C HV/LV kW |  |
| 11 | Impedance voltage at 75°C and CMR: HV/LV % |  |
| 12 | Demonstrable zero phase sequence impedance at 75°C and on normal tapping, referred to HV side, at 10% full load current in each phase winding: |  |
|  | (a) HV/LV Ω per phase |  |
|  | (b) HV-N (Earthing Transformers only) Ω per phase |  |
| 13 | Calculated zero phase sequence impedance at 75°C and on normal tapping referred to HV side, assuming rated voltage (single phase) applied between line terminals and neutral: |  |
|  | (a) ZH Ω per phase (approx.) |  |
|  | (b) ZL Ω per phase (approx.) |  |
|  | (c) ZT/N Ω per phase (approx.) |  |
|  | (d) ZH + ZL Ω per phase (approx.) |  |
| 14 | Proportion of sum of iron and copper losses at CMR which will be supplied during temperature rise test % |  |

SCHEDULE E

TECHNICAL PARTICULARS AND GUARANTEES

PART II

(To be completed by Tenderer)

FOR EARTHING/AUXILIARY TRANSFORMERS

| Item No | Description |  |
| --- | --- | --- |
| 15 | Guaranteed sound level ("A" weighting) dB |  |
|  | Tank and Cooler |  |
| 16 | Tank material |  |
| 17 | Thickness of tank: |  |
|  | (a) Sides mm |  |
|  | (b) Base mm |  |
|  | (c) Cover mm |  |
| 18 | Thickness of radiator plates and/or cooling tubes mm |  |
| 19 | Total oil required (including cooling system) litre |  |
| 20 | Volume of oil to be removed: |  |
|  | (a) to gain access to core earthing link litre |  |
|  | (b) to effect in situ change of HV bushing litre |  |
|  | (c) to effect in situ change of LV bushing litre |  |
| 21 | Total volume of conservator(s) litre |  |
| 22 | Volume of oil in conservator between highest and lowest visible levels litre |  |
| 23 | Filling medium for transport |  |
| 24 | Total weight as installed in service, including all fittings and oil |  |
|  | (a) quotation estimate Tonne |  |
|  | (b) final design calculation Tonne |  |
| 25 | Weight arranged for transport: |  |
|  | (a) excluding vehicle Tonne |  |
| 26 | Weight of each bushing indicator: |  |
|  | (a) HV kg |  |
|  | (b) LV kg |  |
|  | (c) Neutral kg |  |
| 27 | Weight of silica-gel breather charge (if applicable) kg |  |

SCHEDULE E

TECHNICAL PARTICULARS AND GUARANTEES

PART II

(To be completed by Tenderer)

FOR EARTHING/AUXILIARY TRANSFORMERS

| Item No | Description |  |
| --- | --- | --- |
|  | Neutral Coupler |  |
| 28 | Insulation Level |  |
| 29 | Cooling |  |
| 30 | Rated Power |  |
| 31 | Rated Voltage |  |
| 32 | Rated Continuous Current |  |
| 33 | Rated Continuous Neutral Current |  |
| 34 | Winding Connections: - HV winding - LV Winding |  |
| 35 | Zero sequence impedance per phase |  |
| 36 | Total Oil required |  |
| 37 | Filling medium for transport |  |
| 38 | Total weight as installed in service |  |
| 39 | Total weight for transport |  |

SCHEDULE E

TECHNICAL PARTICULARS AND GUARANTEES

PART III

QUALITY OF MATERIALS

(Relevant details to be completed by Tenderer)

| Material | Specification of Analysis |
| --- | --- |
| Aluminium Alloys: |  |
| i) Castings for current-carrying parts |  |
| ii) Other purposes |  |
| Gaskets for oil-tight joints |  |

SCHEDULE F

DRAWINGS

(To be completed by Tenderer)

The following is a list of Drawings which shall be submitted by the Tenderer with his Tender:

| Description | Tenderer's Drg. No. |
| --- | --- |
| - A preliminary outline giving side and elevation and plan of the tank including tapchanger, terminals/cable boxes and controlling equipment. The principal dimensions shall be given and parts to be removed for transport shall be indicated. NOTE: The outline drawing shall indicate the worst case civil interfaces which cannot be exceeded during the contract. |  |
| (b) A preliminary diagram plate and rating plate. |  |

SCHEDULE G

PRICES AND QUANTITIES FOR DEFINITE WORKS

(RELEVANT DETAILS TO BE COMPLETED BY TENDERER)

The Prices (other than the provisional sums) entered below (items 13 to16) include the whole of the definite work specified in the Specification.

| 1 | Section(s) for purposes of payment(s) and taking over |  |  |
| --- | --- | --- | --- |
| 2 | Subtation Name |  |  |
| 3 | Number of 132kV/Lower Voltage Transformers required |  |  |
| 4 | Continuous maximum rating | MVA |  |
| 5 | Rated higher voltage between phases | kV |  |
| 6 | Rated lower voltage between phases | kV |  |
| 7 | Number of Earthing/Auxiliary Transformers required Type: Earthing transformer with resistance earthed system |  |  |
| 8 | Continuous maximum rating | kVA |  |
| 9 | Rated higher voltage between phases | kV |  |
| 10 | Rated lower voltage between phases (no load) | kV | 0.400/0.230 Three phase four wire |
| 11 | Number of Neutral Earthing Reactor required |  |  |
| 12 | Rated voltage | kV |  |
| 13 | (a) Price of each 3 phase 132kV/Lower Voltage transformer (Temperature rise test and impulse tests to be quoted as an extra in Schedule H) | £ |  |
|  | Loss Evaluation (Each 132kV/Lower Voltage transformer): |  |  |
|  | (b) Schedule D No load loss kWx£ | £ |  |
|  | (c) Schedule D Load loss kWx£ | £ |  |
| 14 | Total evaluated cost (a+b+c) | £ |  |
| 15 | Price of each Earthing/Auxiliary Transformer | £ |  |
| 16 | Total evaluated cost of Section | £ |  |
| 17 | Price for delivery and off-loading each transformer | £ |  |
| 18 | Price for installation and commissioning each transformer | £ |  |

SCHEDULE H

PRICES FOR WORK AT THE OPTION OF Electricity North West

(RELEVANT DETAILS TO BE COMPLETED BY TENDERER)

| Item | Description | Price of each item £ |
| --- | --- | --- |
| 1 | Extra for: |  |
|  | (a) For temperature rise tests per transformer |  |
|  | (b) For impulse tests per transformer |  |
|  | (c) For each additional shot, impulse test |  |
|  | (d) For discharge test |  |
| 2 | Extra for: |  |
|  | (a) Supply and fitting of anti-vibration pads (transformers only) | To be included |
|  | (b) Noise enclosure if required from outset |  |
| 3 | Extra for Tap Change Control Equipment |  |
|  | (a) For standby Control Panel complete |  |
|  | (b) For supply of Loose Equipment only. |  |

SCHEDULE I

ACCESS FACILITIES TO THE SITE

(TO BE COMPLETED BY ELECTRICTY NORTH WEST)

| 1 | Section |  |
| --- | --- | --- |
| 2 | Subtation Name |  |
| 3 | Ordnance Survey 132kV/Lower Voltage reference |  |
| 4 | Whether direct road access is available to the site | Yes/No |
| 5 | Approximate distance from the site to the nearest road |  |
| 6 | Whether rail transport is required | Yes/No |
| 7 | Whether there is a railway siding near to the site | Yes/No |
| 8 | Nearest railway goods station and approximate distance from the site to the nearest goods station |  |
| 9 | Whether there are any known transport restrictions in the immediate locality of site |  |

SCHEDULE J

PROPOSED DEPARTURES FROM THIS SPECIFICATION

(RELEVANT DETAILS TO BE COMPLETED BY TENDERER)

| Ref. | Proposed departures from the requirements of this specification | Details of alternative proposals |
| --- | --- | --- |

SCHEDULE K

LIST OF SPECIAL TOOLS AND EQUIPMENT TO BE PROVIDED

(RELEVANT DETAILS TO BE COMPLETED BY TENDERER)

| Item No | Description |
| --- | --- |

Appendix B

SELF CERTIFICATION CONFORMANCE DECLARATION

SECTION BY SECTION CONFORMANCE WITH SPECIFICATION

The manufacturer shall declare conformance or otherwise, section by section, using the following levels of conformance declaration codes.

Conformance declaration codes

| N/A = | Section is not applicable/appropriate to the product/service |
| --- | --- |
| C1 = | The product/service conforms fully with the requirements of this section |
| C2 = | The product/service conforms partially with the requirements of this section |
| C3 = | The product/service does not conform to the requirements of this section |
| C4 = | The product/service does not currently conform to the requirements of this section, but the manufacturer proposes to modify and test the product in order to conform. |

| Manufacturer: |  |
| --- | --- |
| Product/Service Description: |  |
| Product /Service Reference : |  |

Assessor

| Name: | Company: | Signature: | Date: |
| --- | --- | --- | --- |

| Section | Requirement | Conformance Code | Remarks (Must be completed if Conformance Code is not C1) |
| --- | --- | --- | --- |
| 4.1 | Standards |  |  |
| 4.2 | Compliance With Specification |  |  |
| 4.3 | Safety |  |  |
| 4.4 | Environmental Conditions |  |  |
| 4.5 | System Conditions |  |  |
| 4.6 | Electrical Clearances |  |  |
| 4.7 | Terminal Points of Contract |  |  |
| 4.8 | Design of Equipment |  |  |
| 4.9 | Rating of Ancillaries |  |  |
| 4.10 | Quality of Material |  |  |
| 4.11 | Fasteners and Fittings |  |  |
| 4.12 | Interchangeability |  |  |
| 4.13 | Insulating Oil |  |  |
| 4.14 | Tests |  |  |
| 4.15 | Transformer Management Systems |  |  |
| 5.1 | Product not to be changed |  |  |
| 5.2 | Electricity North West Technical Approval |  |  |
| 5.3 | Quality Assurance |  |  |
| 5.4 | Formulation |  |  |
| 5.5 | Identification Marking |  |  |
| 5.6 | Manufacturers already Approved |  |  |
| 5.7 | Product Conformity |  |  |
| 6 | Requirements for Type and Routine Testing |  |  |
| 7.1.1 | Windings |  |  |
| 7.1.2 | General |  |  |
| 7.1.3 | Cooling |  |  |
| 7.1.4 | Parallel Operation |  |  |
| 7.2 | Ratings and Loadings |  |  |
| 7.3 | Voltage Ratio and Taps |  |  |
| 7.4 | Winding Connections and Vector Group |  |  |
| 7.5 | Losses and Evaluation of Losses |  |  |
| 7.6 | Impedances |  |  |
| 7.7 | Duty under Fault Conditions |  |  |
| 7.8 | Magnetic Circuit |  |  |
| 7.9 | Flux Density |  |  |
| 7.10 | Vibration |  |  |
| 7.11 | Noise |  |  |
| 7.12 | Terminal Connections |  |  |
| 7.13 | Tanks and Coolers |  |  |
| 7.14 | Voltage Control |  |  |
| 8.1 | General |  |  |
| 8.2 | Electrical Characteristics |  |  |
| 8.3 | Short Time Current Rating |  |  |
| 8.4 | Tanks and Fittings |  |  |
| 8.5 | Lower Voltage Windings |  |  |
| 8.6 | Multicore Cable Connections |  |  |
| 9.1 | General |  |  |
| 9.2.1 | General Requirements |  |  |
| 9.2.2 | Laminations |  |  |
| 9.2.3 | Structural Framework |  |  |
| 9.2.4 | Core Bands |  |  |
| 9.2.5 | Flux Density |  |  |
| 9.3.1 | General Requirements |  |  |
| 9.3.2 | Winding |  |  |
| 9.3.3 | Connections |  |  |
| 9.3.4 | Terminal Markings |  |  |
| 9.4.1 | General Requirements |  |  |
| 9.4.2 | Earthing of Core Clamping Structure |  |  |
| 9.4.3 | Earthing of Magnetic Circuits |  |  |
| 9.4.4 | Earthing of Coil Clamping Ring |  |  |
| 9.4.5 | Size of Earthing Connection |  |  |
| 9.5 | Drying Out |  |  |
| 10.1 | Safety and Clearances |  |  |
| 10.2 | Transformer Tanks |  |  |
| 10.3 | Conservator Tanks and Breathers |  |  |
| 10.4 | Earthing Connections |  |  |
| 10.5 | Pressure Relief Devices |  |  |
| 10.6 | Joints and Gaskets |  |  |
| 10.7 | Valves and Plugs |  |  |
| 10.8 | Oil Level Indicators |  |  |
| 10.9 | Winding Temperature Indicators |  |  |
| 10.10 | Gas and Oil Actuated Relays |  |  |
| 10.11 | Rating, Diagram and Valve Plates |  |  |
| 10.12 | Cleaning and Painting |  |  |
| 11.1 | General Requirements |  |  |
| 11.2 | On-load Tap Changers (OLTC) |  |  |
| 11.3 | OLTC Driving Mechanism |  |  |
| 11.4 | Off-Circuit Tap Changers |  |  |
| 11.5 | Voltage Control |  |  |
| 12.1 | General Requirements |  |  |
| 12.2.1 | Particular Requirements |  |  |
| 12.2.2 | Construction |  |  |
| 12.2.3 | Temperature Limits |  |  |
| 12.2.4 | Porcelains and Associated Gaskets |  |  |
| 12.2.5 | Paint and Other Protection |  |  |
| 12.2.6 | Mounting of Bushings |  |  |
| 12.3.1 | Particular Requirements |  |  |
| 12.3.2 | Ratings |  |  |
| 12.3.3 | Construction |  |  |
| 12.3.4 | Venting, Drainage and Earthing |  |  |
| 12.3.5 | Disconnecting Chambers |  |  |
| 12.4 | Current Transformers |  |  |
| 13.1 | General Requirements |  |  |
| 13.2 | Radiators |  |  |
| 13.3 | Cooling Banks |  |  |
| 13.4 | Oil Pipes and Flanges |  |  |
| 13.5 | Oil Pumps |  |  |
| 13.6 | Air Blowers |  |  |
| 13.7 | Motors |  |  |
| 13.8 | Cooler Control |  |  |
| 14.1 | General Particulars |  |  |
| 14.2 | Common Requirements |  |  |
| 14.3 | Electrical Control of Cooling Plant |  |  |
| 14.4.1 | General |  |  |
| 14.4.2 | Local/Standby Control |  |  |
| 14.4.3 | Independent Automatic Voltage Control |  |  |
| 14.4.4 | Voltage Regulating Device CT |  |  |
| 14.4.5 | Indications |  |  |
| 14.5.1 | General |  |  |
| 14.5.2 | Oil Temperature Indicators |  |  |
| 14.5.3 | Winding Temperature Indicators |  |  |
| 14.5.4 | Remote Indication of Winding Temperature |  |  |
| 14.6.1 | General Requirements |  |  |
| 14.6.2 | Earthing |  |  |
| 14.6.3 | Ventilation and Heating |  |  |
| 14.6.4 | Fittings - General |  |  |
| 14.7.1 | General Requirements |  |  |
| 14.7.2 | Interconnecting Cables |  |  |
| 14.7.3 | Small Wiring |  |  |
| 15.1 | General Requirements |  |  |
| 15.2.1 | General |  |  |
| 15.2.2 | Material Tests |  |  |
| 15.2.3 | Test Certificates |  |  |
| 15.3.1 | General |  |  |
| 15.3.2 | General Test Requirements |  |  |
| 15.4 | Rejection of Plant |  |  |
| 15.5 | 132kV/Lower Voltage Transformer - Summary of Tests |  |  |
| 15.6 | 132kV/Lower Voltage Transformer - Details of tests during manufacture |  |  |
| 15.7 | 132kV/Lower Voltage Transformer - Site commissioning tests |  |  |
| 15.8 | Earthing/Auxiliary Transformer - Summary of Tests |  |  |
| 15.9 | Earthing/Auxiliary Transformer - Details of tests during manufacture |  |  |
| 15.10 | Earthing/Auxiliary Transformer - Site commissioning tests |  |  |
| 15.11 | Test certificates and records to be supplied |  |  |
| 16 | Drawings |  |  |
| 17 | Assembly, Operating and Maintenance Instructions |  |  |
| 18 | Models and Samples |  |  |
| Schedule B | Commencement date and Dates of Readiness for Inspection, Testing, Delivery , Access to Site and Completion Definite Work |  |  |
| Schedule C1 | Manufacturers and Places of Manufacture, Testing and Inspection – 132kV/Lower Voltage Transformer |  |  |
| Schedule C2 | Manufacturers and Places of Manufacture, Testing and Inspection – Earthing/Auxiliary Transformer |  |  |
| Schedule E Part I | Technical Particulars and Guarantees for 132kV/Lower Voltage Transformers |  |  |
| Schedule E Part II | Technical Particulars and Guarantees for Earthing/Auxiliary Transformers |  |  |
| Schedule E Part III | Quality of Materials |  |  |
| Schedule F | Drawings |  |  |
| Schedule G | Prices and Quantities for Definite Works |  |  |
| Schedule H | Prices for Works at the Option of Electricity North West |  |  |
| Schedule J | Proposed Departures from this Specification |  |  |
| Schedule K | List of Special Tools and Equipment to be Provided |  |  |

Additional Notes: