APPENDIX B - GENERAL PARTICULARS OF DEFINITE WORK

(Relevant details to be completed by Purchaser)

| Item No. | Description | Value | Detail |
| --- | --- | --- | --- |
| 1 | Sub Station Name and Address |  |  |
| 2 | Number of Units Required |  |  |
| 3 | Voltage Ratio | kV | 11/6.6 |
| 4 | Vector Group |  |  |
| 5 | Continuous Maximum Rating | MVA |  |
| 6 | Impedance | % |  |
| 7 | Highest Voltage for Equipment | Um (r.m.s.) kV | 12 |
| 8 | Rated Lightning Impulse Withstand Voltage | kV (peak) | 75 |
| 9 | Power Frequency Withstand Voltage | kV (r.m.s.) | 28 |
| 10 | Sound Power Level | (dB(A)) |  |
| 11 | Iron Loss | (W) |  |
| 12 | Copper Loss | (W @ 75º C) |  |
| 13 | Protection |  |  |

appendix C - Technical Details

| Item No. | Item | 3MVA | 6MVA |
| --- | --- | --- | --- |
| 1 | Guaranteed No-Load Loss 11kV (watts) |  |  |
| 2 | Guaranteed Load Loss @ 75oC, 11kV (watts) |  |  |
| 3 | Impedance @ 75oC, 11kV (% on rating) (i) On Nominal Tap (ii) On Maximum Tapping (iii) On Minimum Tapping |  |  |
| 4 | Maximum Flux Density in any Magnetic Component (Tesla) |  |  |
| 5 | Type of Core Steel Used Weight, thickness, grade? |  |  |
| 6 | Core Construction Details e.g. step lapped, bolted, banded etc |  |  |
| 7 | 11kV Windings (i) Type (ii) Conductor Material (ii) Insulation Type |  |  |
| 8 | 6.6kV Windings (i) Type (ii) Conductor Material (ii) Insulation Type |  |  |
| 9 | Current Density (i) In 11kV Winding (A/mm2 ) (ii) In 6.6kV Winding (A/mm2 ) |  |  |
| 10 | Type of Tap Changer e.g. rotary, linear etc. |  |  |
| 11 | Tank Details e.g. welded steel, corrugated etc. |  |  |
| 12 | Guaranteed Sound Power Level dB(A) |  |  |
| 13 | Quantity of Oil (litres) |  |  |
| 14 | Weight of Tank and Fittings (kg) |  |  |
| 15 | Weight of Core and Winding Assembly (kg) |  |  |
| 16 | Weight of Copper (kg) |  |  |
| 17 | Tank Construction - Material - Thickness of Sides - Thickness of Base - Thickness of Cover |  |  |
| 18 | Type of Radiators Bolt on or integral with tank |  |  |
| 19 | Overall Dimensions - Length - Width - Height |  |  |
| 20 | Oil Preservation System e.g. free breathing, sealed, hermetically sealed |  |  |
| 21 | Oil Head Space Filler Material |  |  |
| 22 | Tender Drawing Reference Number |  |  |

appendix D - Schedules

SCHEDULE D1 - SUB-CONTRACTORS

| Item | Manufacturer | Place of Manufacture |
| --- | --- | --- |

SCHEDULE D2 - Departures from specification

Tenderer to state details of any departure from the requirements of this specification

| Section and Clause No | Proposed departures from the requirements of this specification, with details of alternative proposals |
| --- | --- |

Appendix E

SELF CERTIFICATION CONFORMANCE DECLARATION

CLAUSE BY CLAUSE CONFORMANCE WITH THIs ES

The manufacturer shall declare conformance or otherwise, clause by clause, using the following levels of conformance declaration codes.

Conformance declaration codes

| N/A = | Clause is not applicable/appropriate to the product/service |
| --- | --- |
| C1 = | The product/service conforms fully with the requirements of this clause |
| C2 = | The product/service conforms partially with the requirements of this clause |
| C3 = | The product/service does not conform to the requirements of this clause |
| C4 = | The product/service does not currently conform to the requirements of this clause, but the manufacturer proposes to modify and test the product in order to conform. |

| Manufacturer : |  |
| --- | --- |
| Product/Service Description : |  |
| Product /Service Reference : |  |

Assessor

| Name | Company | Signature | Date |
| --- | --- | --- | --- |

| Clause/Sub-Clause |  | Requirement | Conformance Code | Remarks (Must be completed if Conformance Code is not C1) |
| --- | --- | --- | --- | --- |
| 4 |  | Compliance with Standards |  |  |
| 5 | 1 | Product not to be changed |  |  |
| 5 | 2 | Electricity North West Technical Approval |  |  |
| 5 | 3 | Quality Assurance |  |  |
| 5 | 4 | Formulation |  |  |
| 5 | 5 | Identification Marking |  |  |
| 5 | 6 | Minimum Life Expectancy |  |  |
| 5 | 7 | Product Conformity |  |  |
| 6 | 1 | Requirement for Type Tests at the Suppliers’ Premises |  |  |
| 6 | 2 | Requirement for Routine Tests at the Suppliers’ Premises |  |  |
| 7 | 1 | General |  |  |
| 7 | 2 | System |  |  |
| 7 | 3 | Rated Power |  |  |
| 7 | 4 | Rated Voltage |  |  |
| 7 | 5 | Tapping Range |  |  |
| 7 | 6 | Tapping Methods |  |  |
| 7 | 7 | Windings and Connections |  |  |
| 7 | 8 | Losses |  |  |
| 7 | 9 | Sound Power Level |  |  |
| 7 | 10 | Impedances |  |  |
| 7 | 11 | Insulation Levels |  |  |
| 7 | 12 | Flux Density |  |  |
| 7 | 13 | Protection |  |  |
| 8 | 1 | Rating and Connection Plates |  |  |
| 8 | 2 | Marking of Terminals |  |  |
| 9 |  | Insulating Oil |  |  |
| 10 | 1 | General Construction |  |  |
| 10 | 2 | Tank Covers |  |  |
| 10 | 3 | Radiators |  |  |
| 10 | 4 | Gaskets |  |  |
| 10 | 5 | Fittings |  |  |
| 10 | 6 | Connections and Cable Terminations |  |  |
| 10 | 7 | Earthing Bar for use with Inter-System Transformers |  |  |
| 11 |  | Cleaning and Painting |  |  |
| 12 |  | Tests |  |  |
| 13 |  | Tender Information |  |  |
| 14 |  | Drawings |  |  |
| 15 |  | Tools and Equipment |  |  |
| 16 |  | Training |  |  |
| Appendix A |  | Ratings |  |  |
| Appendix C |  | Technical Details to be Completed by Tenderer |  |  |
| Appendix D |  | Schedules to be Completed by the Tenderer |  |  |

Additional Notes:

Appendix F – Earthing bar and gland plate example